# FLEET SPECIFICATIONS

| N64270 - 1975 Cessna 172M | ## IFR Day/Night |
| --- | --- |
| Serial No: 17265142 | Usable Fuel: 38 gals. |
| HP: 150 Engine: Lycoming 0-320 | Fuel Type: 80/87 (Red) or 100LL (Blue) |
| Ramp Wt: 2300 / Gross Wt. (Takeoff & Landing): 2300 | *Oil Qty: 6-8 qts. |
| Basic Empty Wt: 1438 \ Useful Load: 862 lbs | Oil Type: 50wt. Summer / 40wt. Winter |
| Moment: 57,253.6 (C.G 39.81) | Tire Pressure (psi): 35 Nose / 32 Main |
| Equip: #1Garmin 430W IFR Color GPS/NAV/COM, #2 King 175 , Dual GS, Marker Beacons, 4 Place Intercom, Wing Strobes |  |
| N734GX - 1977 Cessna 172N | ## IFR Day/Night |
| Serial No: 17268853 | Usable Fuel: 38 gals. |
| HP: 160 Engine: Lycoming 0-320 | Fuel Type: 100LL (Blue) |
| Ramp Wt: 2400 / Gross Wt. (Takeoff & Landing): 2400 | *Oil Qty: 5-7 qts. |
| Basic Empty Wt: 1500 \ Useful Load: 900 lbs | Oil Type: 50wt. Summer / 40wt. Winter |
| Moment: 59,665.5 (C.G 39.78) | Tire Pressure (psi): 35 Nose / 32 Main |

Equip: Dual NAV/COMS with IFR GPS, ADS-B in/out Status 1090, Autopilot with ALT hold 4PL intercom

| N739WM – 1978 Cessna 172N | ## IFR Day/Night |
| --- | --- |
| Serial No: 17270865 | Usable Fuel: 50 gals. |
| HP: 180 Engine: Lycoming 0-360 | Fuel Type: 100LL (Blue) |
| Ramp Wt: 2550 / Gross Wt. (Takeoff & Landing): 2550 | *Oil Qty: 6-8 qts. |
| Basic Empty Wt: 1491.5 \ Useful Load: 1058.5 lbs | Oil Type: 50wt. Summer / 40wt. Winter |
| Moment: 56581.34 (C.G 37.94) | Tire Pressure (psi): 40 Nose / 35 Main |

Equip: Garmin 340 Audio Panel w/4 place Intercom, #1 Garmin 430W GPS/NAV/COM, #2 King 155A NAV/COM, GS and Marker Beacons

REVISED: 09/16/2019

*Note all Aircraft use Aeroshell Oils 100W Plus for Summer / 80W Plus for Winter

1903 Terminal Drive    	 -    	Richland, WA.        	-      	Phone 509-946-2515